Name & Student ID

Boxuan YU  20513790

Title

Writing Robot project

Software Description

The developed software converts the contents of a plain-text file (test.txt) into G-code and streams the commands to a writing robot. In PC-only mode the same G-code is printed to the console and logged into an output/output.gcode file, which can be inspected or replayed later.

The program uses a stroke-based single-line vector font stored in SingleStrokeFont.txt. Each character is represented by a sequence of points with integer coordinates (font units) and a pen flag indicating pen-up or pen-down. At start-up, the font file is read once and all strokes are stored in an internal buffer of StrokePoint structures. Individual glyphs are identified by their ASCII code and located using index ranges into this buffer.

After the font is loaded, the main() routine opens a serial channel (or emulator) via the serial.c layer, performs the wake-up handshake and moves the robot to a safe origin position with the pen raised. The user is then prompted to enter a character height in millimetres. The input is checked and clamped to the assignment range of 4–10 mm. From this height a scale factor is computed to map the font’s reference units into real-world millimetres.

The text rendering is handled by render_text_file(), which opens test.txt, reads it line by line and converts each printable character into G-code. New-line characters cause the writing cursor to reset in X and move down by one line spacing (height + 5 mm). For each character, find_glyph() returns the corresponding stroke range inside the global font buffer; draw_glyph() then walks through the strokes, applies scaling and coordinate transforms, and emits the appropriate sequence of:

- rapid positioning moves (G0) when the pen is up,

- writing moves (G1) when the pen is down,

- pen commands (S0 for pen up, S1000 for pen down).

Low-level G-code emission is centralised in send_move(), which formats a single G0/G1 line, sends it through the serial interface and simultaneously mirrors it into output/output.gcode. This guarantees that the robot and the log file always receive identical motion commands. A simple workspace check in send_move() ensures that generated coordinates remain within the specified rectangle used during testing.

When the complete text file has been processed, main() calls finalise_motion() to raise the pen, return the robot to the origin and stop the spindle, then closes the serial port. The final system therefore provides a complete pipeline from ASCII text, through stroke-based font interpretation and scaling, to runnable G-code suitable for the drawing robot or for offline inspection via the generated output.gcode file.

(Maximum 1 page)

Project Files

main.c
Top-level control of the program. Handles serial initialisation, user interaction for character height, overall error checking and calls into the text rendering engine.

glyph_store.c / glyph_store.h
Implements the storage of the stroke-based font. Defines the StrokePoint structure and provides functions to load the font file, search for a glyph by ASCII code and expose the internal stroke buffer to other modules.

glyph_painter.c / glyph_painter.h
Contains the functions that transform glyph strokes into robot motion. Responsible for bounds checking in workspace coordinates, scaling from font units to millimetres, and composing G0/G1/S… command lines. All G-code passes through this module, which also writes to output/output.gcode.

Text_engine.c / Text_engine.h
Implements the text-file processing logic. Opens test.txt, reads each line, interprets new-line characters and passes individual characters to the glyph painter at the correct cursor position. Manages the current X/Y text cursor and vertical line spacing.

serial.c / serial.h
Provides a thin wrapper around the low-level RS232 library for this project. Contains PC-mode implementations of CanRS232PortBeOpened(), CloseRS232Port(), PrintBuffer(), WaitForReply() and WaitForDollar(). In emulator mode these functions print to the console instead of a physical COM port.

rs232.c / rs232.h
External RS232 communication library supplied with the assignment. Used by serial.c when hardware serial is enabled.

SingleStrokeFont.txt
Stroke-based vector font definition. Each line stores x, y and pen-state values. The file is parsed by glyph_store.c.

test.txt
Input text file to be written by the robot. Its entire contents are rendered by Text_engine.c.

output/output.gcode
Automatically generated log file containing every G-code motion command emitted during a run. Useful for debugging or offline simulation.

(Maximum 1 page)

Key Data Items

| Name | Data type | Rationale |
| --- | --- | --- |
| StrokePoint | struct | Represents one point of a stroke in font units: integer x, y and pen flag (up/down). |
| font_buffer | StrokePoint | Global array holding all strokes loaded from SingleStrokeFont |
| font_size | int | Number of valid entries in font_buffer[]; used for bounds checking. |
| glyph_index | int | Lookup table giving the start index of each ASCII glyph in font_buffer[]. |
| glyph_count | int | Number of stroke points for each ASCII glyph. |
| height | int | User-selected character height in millimetres (4–10 mm). |
| cursor_x, cursor_y | float | Current text cursor position in the robot workspace, updated after each character. |
| line_spacing | float | Vertical distance between lines, defined as height_mm + 5.0f |
| gcode_file | file | File handle for output/output.gcode, used to mirror all emitted G-code. |
| input_line | char | Temporary buffer for reading lines from test.txt. |
| ascii_code | int | Stores the ASCII code of the current character during rendering. |

Extend table as required

Functions

Function: load_font_file

Parameters:
const char *filepath – path to the single-stroke font file (for example "SingleStrokeFont.txt").

Description:
Reads the stroke-based font file and loads all stroke points into internal buffers. The function parses glyph header lines and stroke lines, fills a global array of StrokePoint structures, and builds an index table so that each ASCII code can be mapped to a contiguous block of strokes.

Return value:
Returns 0 if the font file is opened and parsed successfully. Returns a non-zero value if the file cannot be opened, if the stroke or glyph buffers overflow, or if no valid glyph data are found.

Function: find_glyph

Parameters:
int ascii_code – ASCII code of the character to look up.
int *start_index – pointer that receives the first stroke index of the glyph.
int *count – pointer that receives the number of strokes belonging to the glyph.

Description:
Searches the internal glyph index table for the specified ASCII code. When a match is found, the function returns the range of indices in the global stroke buffer that correspond to this glyph. This information is later used by the drawing routine to access only the relevant stroke points.

Return value:
Returns 0 if the glyph for ascii_code is found. Returns a non-zero value if no glyph entry exists for that character.

Function: get_stroke_buffer

Parameters: None.

Description:
Provides read-only access to the internal stroke buffer that stores all StrokePoint entries loaded from the font file. Other modules use this pointer together with the index range returned by find_glyph to reconstruct the geometry of each glyph.

Return value: Returns a pointer to the first element of the global stroke buffer.

Function: draw_glyph

Parameters:
int ascii_code – ASCII code of the character to be drawn.
float *cursor_x – pointer to the current X position of the text cursor in millimetres.
float *cursor_y – pointer to the current Y position of the text cursor in millimetres.
float scale – scale factor that converts font units into millimetres.

Description:
Draws a single glyph at the current text cursor position using the stroke-based font. The function calls find_glyph and get_stroke_buffer to obtain the stroke range for the requested character, checks that the scaled glyph will remain inside the predefined workspace, and then sends the corresponding G0, G1 and pen commands through the serial layer. Pen-up and pen-down states are handled so that only the intended stroke segments are plotted.

Return value:
Returns 0 if the glyph is processed successfully (including the case where an unsupported character is reported and skipped). Returns a non-zero value if the glyph would exceed the drawing area or another drawing error occurs.

Function: render_text_file

Parameters:
const char *filepath – path to the input text file to be drawn.
float height_mm – target character height in millimetres.

Description:
Opens the text file and converts its contents into a sequence of glyph drawing operations. The function computes a scale factor from height_mm, maintains a text cursor in workspace coordinates, and implements simple layout rules for character spacing and line spacing. Printable characters are passed to draw_glyph. Spaces advance the cursor horizontally without drawing, and newline characters move the cursor down to the start of the next line.

Return value: Returns 0 if the file is processed and rendered without errors. Returns a non-zero value if the text file cannot be opened or if a drawing error occurs.

Function: PrintBuffer

Parameters:
char *buffer – pointer to a null-terminated string containing one line of G-code or a command to be transmitted.

Description:
Sends a single line of text to the robot controller (or emulator). In serial mode the string is written to the COM port; in PC mode it is printed to the console. In the extended version it also appends each line to an output G-code file so that the full command stream can be inspected after the run.

Return value: Returns 0 after the line has been sent (and logged, if enabled).

Function: WaitForReply

Parameters: None.

Description:
Waits for a reply from the robot after a G-code command has been sent. In serial mode it continuously polls the port until an “ok” response is received or equivalent confirmation is detected in the incoming characters. In emulator mode this function returns immediately because no real hardware handshake is required.

Return value: Returns 0 when the expected reply has been received and it is safe to send the next command.

Function: WaitForDollar

Parameters: None.

Description:
Used during start-up to perform the initial handshake with the robot controller. The function waits until a dollar sign ('$') or a suitable ready message is received from the serial port, indicating that the robot firmware is awake and ready to accept G-code. In emulator mode the function returns immediately.

Return value: Returns 0 once the ready signal has been detected.

Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |
| load_font_file | Load font under normal conditions | SingleStrokeFont present in working directory | Function returns 0; console prints the number of glyphs and strokes loaded; no error messages |
| find_glyph | Lookup a valid printable character | ASCII 'A', 'B', '0', | Function returns 0, and both start_index and count contain non-zero values consistent with the font file. |
| find_glyph | Lookup unsupported character | ASCII values do not present in the font | Function returns non-zero; caller skips the glyph without drawing. |
| get_stroke_buffer | Access stroke buffer after successful font load | Call immediately after load_font_file |  |

Extend table as required. Note that ‘Function’ includes main()

AI Statement

Flowchart(s)

Included as separate pdf